Faculty of New Sciences and Technologies, University of Tehran

"In the name of God"

Material Safety Data Sheet

Product name: Dichloroethane

Creation Date: 22 April 2021

# Identification

Product Name: Dichloroethane

Chemical Formula: C2H4Cl2

Recommended Use: Laboratory chemical

Uses advised against: Not for food, drug, pesticide, or biocidal product use

# Hazards Identifications

Flammability: Highly flammable liquid and vapor

Health effects – eyes: Causes serious eye irritation

Health effects – skin: Causes skin irritation

Health effects – inhalation: Toxic and respiratory irritation

Health effects – swallowed: Harmful if swallowed

May cause drowsiness or dizziness and cancer

# First-aid Measures

Product in the eye: Rinse immediately with plenty of water, also under the eyelids, for at least 15 minutes. Immediate medical attention is required.

Product on skin: Wash off immediately with plenty of water for at least 15 minutes. Immediate medical attention is required.

Product inhaled:

# Fire-fighting Measures

Suitable Extinguishing Media: Use water spray, alcohol-resistant foam, dry chemical, or carbon dioxide. Cool closed containers exposed to fire with water spray.

# NFPA Hazard Classification

NFPA Health Rating: 2 -   Materials that, under emergency conditions, can cause temporary incapacitation or residual injury.

NFPA Fire Rating: 3 - Liquids and solids that can be ignited under almost all ambient temperature

conditions. Materials produce hazardous

atmospheres with air under almost all ambient temperatures or, though unaffected by ambient temperatures, are readily ignited under almost all conditions.

NFPA Instability Rating: 0 -   Materials that in themselves are normally stable, even under fire conditions.