| Faculty of New Sciences and Technologies, University of Tehran | "In the name of God" |
| --- | --- |

Material Safety Data Sheet

Product name: Acetic Acid

Creation Date: 22 April 2021

Identification

Product Name: Acetic Acid, Ethanoic acid, Ethylic acid

Chemical Formula: C2H4O2 or CH3COOH

Recommended Use: Laboratory chemical

Uses advised against: Not for food, drug, pesticide, or biocidal product use

Hazards Identifications

Flammability: Combustible liquid

Health effects: Causes severe skin burns and eye damage

Preventions:

- Do not breathe dust/fume/gas/mist/vapors/spray

- Wash face, hands, and any exposed skin thoroughly after handling

- Wear protective gloves/protective clothing/eye protection/face protection

- Use only outdoors or in a well-ventilated area

- Keep away from heat/sparks/open flames/hot surfaces. - No smoking

- Keep cool

First-aid Measures

Product in the eye: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing

Product on the skin (or hair): Take off immediately all contaminated clothing. Rinse skin with water/shower Wash contaminated clothing before reuse

Product inhaled: Remove victim to fresh air and keep at rest in a position comfortable for breathing

Fire-fighting Measures

Suitable Extinguishing Media: CO2, dry chemical, dry sand, alcohol-resistant foam. Cool closed containers exposed to fire with water spray.

#### NFPA Hazard Classification

NFPA Health Rating: 3 -   Materials that, under emergency conditions, can cause serious or permanent injury.

NFPA Fire Rating: 2 - Materials that must be moderately heated or exposed to relatively high ambient temperatures before ignition can occur. Materials would not under normal conditions form hazardous atmospheres with air, but under high ambient temperatures or under moderate heating could release vapor in sufficient quantities to produce hazardous atmospheres with air.

NFPA Instability Rating: 0 -   Materials that in themselves are normally stable, even under fire conditions.