Faculty of New Sciences and Technologies, University of Tehran

|  | "In the name of God" |
| --- | --- |

Material Safety Data Sheet

Product name: Ethanol

Creation Date: 22 April 2021

Identification

Product Name: Ethanol

Chemical Formula: C2H5OH or CH3CH2OH

Recommended Use: Laboratory chemical

Uses advised against: Not for food, drug, pesticide, or biocidal product use

Hazards Identifications

Flammability: Extremely flammable liquid

Health effects – eyes: Moderately irritating. Exposure to liquid, vapors, fumes, or mist may irritate. Direct contact may cause irritation, redness, pain, corneal inflammation, and possible corneal damage.

Health effects – skin: Repeated or prolonged contact may result in defatting, redness, pain, itching, inflammation, cracking, and possible secondary infection.

Health effects – inhalation: intoxicating if continuously inhaled for a long time

First-aid Measures

Product in the eye: Flush immediately with water or neutral saline solution for at least 15 minutes. Seek medical attention.

Product on the skin: Remove contaminated clothing and rinse the contaminated area with soap and water. If skin irritation persists seek medical attention.

Product inhaled: Remove victim to fresh air and keep at rest in a position comfortable for breathing.

Fire-fighting Measures

Suitable Extinguishing Media: Use dry chemical, alcohol foam, or carbon dioxide to extinguish the fire. Water may be ineffective but should be used to cool fire-exposed containers, structures and to protect personnel.

#### NFPA Hazard Classification

NFPA Health Rating: 2 -   Materials that, under emergency conditions, can cause temporary incapacitation or residual injury.

NFPA Fire Rating: 3 - Liquids and solids that can be ignited under almost all ambient temperature conditions. Materials produce hazardous atmospheres with air under almost all ambient temperatures or, though unaffected by ambient temperatures, are readily ignited under almost all conditions.

NFPA Instability Rating: 0 -   Materials that in themselves are normally stable, even under fire conditions.