| "In the name of God" |  |
| --- | --- |

Faculty of New Sciences and Technologies, University of Tehran

Material Safety Data Sheet

Product name: Acetone

Creation Date: 22 April 2021

Identification

Product Name: Acetone, Propanone, Dimethyl ketone

Chemical Formula: C3H6O or CH3-CO-CH3 or CH3COCH3

Recommended Use: Laboratory chemicals

Uses advised against: Not for food, drug, or household use

Hazards Identifications

Hazard Statements:

- Highly flammable liquid and vapor

- Causes serious eye irritation.

- May cause drowsiness or dizziness Precautionary statements.

- Keep away from heat, hot surfaces, open flames, sparks. - No smoking.

- Keep container tightly closed.

- Ground/bond container and receiving equipment.

- Avoid breathing mist, spray, vapors.

- Wash exposed skin thoroughly after handling.

- Use only outdoors or in a well-ventilated area.

- Store locked up and keep cool.

First-aid Measures

Product in the eye: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

Product on the skin: Remove/Take off immediately all contaminated clothing. Rinse skin with water/shower.

Product inhaled: Remove person to fresh air and keep comfortable for breathing.

Fire-fighting Measures

For Small Fires Only: Use carbon dioxide, dry powder, or foam.

Special Exposure Hazards: will ignite and burn fiercely in contact with fluorine.

#### NFPA Hazard Classification

NFPA Health Rating: 1 -   Materials that, under emergency conditions, can cause significant irritation.

NFPA Fire Rating: 3 - Liquids and solids that can be ignited under almost all ambient temperature conditions. Materials produce hazardous atmospheres with air under almost all ambient temperatures or, though unaffected by ambient temperatures, are readily ignited under almost all conditions.

NFPA Instability Rating: 0 -   Materials that in themselves are normally stable, even under fire conditions.