| "In the name of God" |  |
| --- | --- |

Faculty of New Sciences and Technologies, University of Tehran

Material Safety Data Sheet

# Product Name: Isopropyl alcohol

Creation Date: 22 April 2021

Identification

Product Name: isopropanol, Isopropyl alcohol

Chemical Formula:  C3H8O or CH3CHOHCH3 or (CH3)2CHOH

Recommended Use: Laboratory chemicals

Uses advised against: Not for food, drug, or biocidal product use

Hazards Identifications

Hazard Statements:

Highly Flammable liquid and vapor [Danger Flammable liquids]

- Causes serious eye irritation [Warning Serious eye damage/eye irritation]

- May cause drowsiness or dizziness [Warning Specific target organ toxicity, single exposure; Narcotic effects]

First-aid Measures

Product in the eye: Flush eyes with plenty of water for at least 15 minutes, occasionally lifting the upper and lower eyelids. Get medical aid.

Product on the skin: Get medical aid. Flush skin with plenty of water for at least 15 minutes while removing contaminated clothing and shoes. Wash clothing before reuse.

Product inhaled: Remove from exposure and move to fresh air immediately. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical aid at once.

Product swallowed: Give conscious victim 2-4 cupfuls of milk or water. Never give anything by mouth to an unconscious person. Get medical aid at once.

Fire-fighting Measures

Extinguishing media: Use water spray to cool fire-exposed containers. Water may be ineffective. Do NOT use straight streams of water.

#### NFPA Hazard Classification

NFPA Health Rating: 1 -  Materials that, under emergency conditions, can cause significant irritation.

NFPA Fire Rating: 3 - Liquids and solids that can be ignited under almost all ambient temperature conditions. Materials produce hazardous atmospheres with air under almost all ambient temperatures or, though unaffected by ambient temperatures, are readily ignited under almost all conditions.

NFPA Instability Rating: 0 -   Materials that in themselves are normally stable, even under fire conditions.