| "In the name of God" Faculty of New Sciences and Technologies, University of Tehran |  |
| --- | --- |

Material Safety Data Sheet

# Product Name: Nitromethane

Creation Date: 22 April 2021

Identification

Product Name: Nitromethane, Nitrocarbol, Nitrometan

Chemical Formula:  CH3NO2

Recommended Use: Laboratory chemicals

Uses advised against: Not for food, drug, or biocidal product use

Hazards Identifications

Hazard Statements:

- Flammable liquid and vapor [Warning Flammable liquids]

- Harmful if swallowed [Warning Acute toxicity, oral]

- A very dangerous fire hazard when exposed to heat, oxidizers, or flame.

- Maybe a CARCINOGEN in humans since it has been shown to cause liver, lung, and glandular cancer in animals.

- May cause headache, weakness, and loss of coordination.

First-aid Measures

Product in the eye: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

Product on the skin: Take off immediately all contaminated clothing. Rinse skin with water/shower Wash contaminated clothing before reuse.

Product inhaled: Remove victim to fresh air and keep at rest in a position comfortable for breathing Immediately

Fire-fighting Measures

Extinguishing media: Use CO2, water spray, or alcohol-resistant foam for extinction.

- Don’t use dry chemical extinguishers on a fire.

#### NFPA Hazard Classification

NFPA Health Rating: 2 -   Materials that, under emergency conditions, can cause temporary incapacitation or residual injury.

NFPA Fire Rating: 3 - Liquids and solids that can be ignited under almost all ambient temperature conditions. Materials produce hazardous atmospheres with air under almost all ambient temperatures or, though unaffected by ambient temperatures, are readily ignited under almost all conditions.

NFPA Instability Rating: 4 -   Materials that in themselves are readily capable of detonation or explosive decomposition or explosive reaction at normal temperatures and pressures.