| Faculty of New Sciences and Technologies, University of Tehran "In the name of God" |  |
| --- | --- |

Material Safety Data Sheet

# Product Name: Nitric Acid

Creation Date: 22 April 2021

Identification

Product Name: Nitric Acid, Hydrogen nitrate, Aqua fortis, Azotic acid

Chemical Formula:  HNO3

Recommended Use: Laboratory chemicals

Uses advised against: Not for food, drug, or biocidal product use

Hazards Identifications

Hazard Statements:

May intensify fire; oxidizer

- It May be corrosive to metals

- Causes severe skin burns and eye damage

- Do not breathe dust/fume/gas/mist/vapors/spray

- Wash face, hands, and any exposed skin thoroughly after handling

- Wear protective gloves/protective clothing/eye protection/face protection

- Use only outdoors or in a well-ventilated area

- Keep away from heat/sparks/open flames/hot surfaces. - No smoking

- Keep/Store away from clothing/ other combustible materials

- Take any precaution to avoid mixing with combustibles

First-aid Measures

Product in the eye: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

Product on the skin: Take off immediately all contaminated clothing. Rinse skin with water/shower Wash contaminated clothing before reuse.

Product inhaled: Remove victim to fresh air and keep at rest in a position comfortable for breathing Immediately call a POISON CENTER or doctor/physician

Product swallowed: Rinse mouth. DO NOT induce vomiting.

Fire-fighting Measures

Extinguishing media: Use CO2, dry chemical, or foam for extinction.

#### NFPA Hazard Classification

NFPA Health Rating: 4 -   Materials that, under emergency conditions, can be lethal.

NFPA Fire Rating: 0 - Materials that will not burn under typical fire conditions, including intrinsically non-combustible materials such as concrete, stone, and sand.

NFPA Instability Rating: 0 -   Materials that in themselves are normally stable, even under fire conditions.

NFPA Specific Notice: OX – Oxidizer - Materials that possess oxidizing properties.