Faculty of New Sciences and Technologies, University of Tehran

"In the name of God"

Material Safety Data Sheet

Product name: Sulfuric Acid

Creation Date: 22 April 2021

# Identification

Product name: Sulfuric Acid

Chemical Formula: H2SO4

# Hazards Identifications

Physical State And Appearance: Odorless, clear to amber, heavy, oily liquid. A pungent odor may exist if certain impurities are present in the acid .

Emergency Overview: Danger! Extremely corrosive. Causes severe burns and / or eye damage .

Mist: Causes respiratory irritation. Harmful if inhaled. Harmful or fatal if swallowed. Reacts violently with water. Concentrated Sulfuric Acid will react with many organic materials and may cause fire due to the heat  of the reaction. Not flammable, but reacts with most metals to form explosive/flammable hydrogen gas.

Health effects – eyes: Immediate pain, severe burns and corneal damage, which may result in permanent blindness.

Health effects – skin: Causes burns, and brownish or yellow stains. Concentrated solutions may cause second or  third degree burns with severe necrosis. Prolonged and repeated exposure to dilute solutions may cause  irritation, redness, pain and drying and cracking of the skin.

Health effects – inhalation: Causes respiratory irritation and at high concentrations may cause severe injury, burns, or death. Effects of exposure may be delayed.

# First-aid Measures

Eye Contact: Immediately flush eyes with running water for a minimum of 20 minutes. Hold eyelids  open during flushing. If irritation persists, repeat flushing. Obtain medical attention IMMEDIATELY. Do not transport victim until the recommended flushing period is completed unless flushing can be continued during transport.

Skin Contact: Immediately flush skin with running water for a minimum of 20 minutes. Start flushing  while removing contaminated clothing. If irritation persists, repeat flushing. Obtain medical attention  immediately. Do not transport victim unless the recommended flushing period is completed or flushing can be continued during transport.

Inhalation: Move victim to fresh air. Give artificial respiration ONLY if breathing has stopped. Do not use mouth-to-mouth method if victim ingested or inhaled the substance: induce artificial respiration with the aid of a pocket mask equipped with a one-way valve or other proper respiratory medical device. Give Cardiopulmonary Resuscitation (CPR) if there is no pulse AND no breathing. Obtain medical attention IMMEDIATELY.

# Fire-fighting Measures

Flammability of the product: Non-flammable

Flash points: Not applicable

Auto-ignition temperature: Not applicable

Flammable limits: Not applicable

Products of thermal decomposition: Oxides of Sulfur

Explosion Hazards :Not flammable but highly reactive. Reacts violently with water with evolution of heat can react with organic  materials explosively. Reacts with many metals to liberate hydrogen gas which can form explosive mixtures with air. Hydrogen can accumulate to explosive concentrations. May ignite other combustible materials.