| "In the name of God" Faculty of New Sciences and Technologies, University of Tehran |  |
| --- | --- |

Material Safety Data Sheet

Product name: Ammonia

Creation Date: 22 April 2021

Identification

Product Name: Ammonia, Azane

Chemical Formula: NH3

Recommended Use: Laboratory chemicals.

Uses advised against: Food, drug, pesticide or biocidal product use.

Hazards Identifications

Hazard Statements:

- Flammable gas [Danger Flammable gases]

- Causes severe skin burns and eye damage [Danger Skin corrosion/irritation]

- Toxic if inhaled [Danger Acute toxicity, inhalation]

- Very toxic to aquatic life [Warning Hazardous to the aquatic environment, acute hazard]

First-aid Measures

Product in the eye: Rinse immediately with plenty of water, also under the eyelids, for at least 15 minutes. Immediate medical attention is required.

Product on the skin: Wash off immediately with plenty of water for at least 15 minutes. Immediate medical attention is required.

Product inhaled: Remove to fresh air. If breathing is difficult, give oxygen. Do not use mouth-to-mouth method if victim ingested or inhaled the substance. Product ingested: Do NOT induce vomiting. Call a physician or Poison Control Centre immediately.

Fire-fighting Measures

Suitable Extinguishing Media: CO2, dry chemical, dry sand, alcohol-resistant foam. Use extinguishing measures that are appropriate to local circumstances and the surrounding environment.

#### NFPA Hazard Classification

NFPA Health Rating: 3 -   Materials that, under emergency conditions, can cause serious or permanent injury.

NFPA Fire Rating: 1 - Materials that must be preheated before ignition can occur. Materials require considerable preheating, under all ambient temperature conditions, before ignition and combustion can occur.

NFPA Instability Rating: 0 -   Materials that in themselves are normally stable, even under fire conditions.