Faculty of New Sciences and Technologies, University of Tehran

|  | "In the name of God" |
| --- | --- |

Instrument Safety Data Sheet

Product name: Orthoxylene

Creation Date: 23 April 2021

Identification

Product name: Orthoxylene

Chemical name: C6H6(CH3)3

Synonyms: Benzene, 1,2 – Dimethyl-;O-imethylbenzene; 1,2  Dimethylbenzene;O-    Methyl toluene.

What are the hazards?

Hazardous substances: Ethyl benzene Risk phrase : R20 Ortho-xylene Risk phrase : R20/21 Harmful by inhalation and in contact with Skin R 38 Irritating to skin

Hazardous label(s):FLAMMABLE LIQUID AND VAPOR. May cause liver and kidney damage. Aspiration hazard if swallowed. Can enter lungs and cause damage cause respiratory  tract irritation.                                                                                                                                                               WARNING! Cause eye irritation and skin irritation. May be harmful if absorbed through the skin or inhaled.

First-aid Measures

Product in the eye: Flush eyes with plenty of water for at least 15 minutes. Then get immediate medical attention.

Product on skin: Wash skin with soap and water for at least 15 minutes while removing contaminated clothing and shoes. Get medical attention, if needed. Thoroughly clean and dry contaminated clothing and shoes before reuse.

Product inhaled: If adverse effects occur, remove to uncontaminated area. Give artificial respiration if not breathing. If breathing is difficult; Oxygen should be administered by qualified personnel. Get immediate medical attention.

Fire-fighting Measures

Flammable class: Severe fire hazard. Vapour /air mixtures are explosive. The vapour is heavier than air. Vapours or gases may ignite at distant ignition sources and flash back. Electrostatic discharges may be generated by flow or agitation resulting in ignition or explosion.

FLASH POINT: 63 ْ F (17 ْ C)

LOWER FLAMEABLE LIMIT: 0. 9%

UPPER FLAMEABLE LIMIT: 6.7 %

AUTOIGNITION: 865 ْF (463 ْC)

Suitable extinguishing media:     Use water spray to cool fire-exposed containers. Use water spray, dry chemical, carbon dioxide, or appropriate foam. regular dry chemical carbon dioxide, water, and regular foam.