Faculty of New Sciences and Technologies, University of Tehran

|  | "In the name of God" |
| --- | --- |

Instrument Safety Data Sheet

Product name: 1,2-Dichloroethane

Creation Date: 22 April 2021

Identification

Product name: 1,2-Dichloroethane

Synonyms: Ethylene dichloride; Ethylene chlorid

Recommended Use: Laboratory chemicals.

Uses advised against: Not for food, drug, pesticide or biocidal prod

What are the hazards?

Hazard Statements: Highly flammable liquid and vapor, Harmful if swallowed, Causes skin irritation, Causes serious eye irrita

Inhalation: IF INHALED, Remove victim to fresh air and keep at rest in a position comfortable for breathing Call a POISON CENTER or doctor/physician

Skin: If skin irritation occurs: Get medical advice/attention. IF ON SKIN (or hair), Take off immediately all contaminated clothing. Rinse skin with water/shower Wash contaminated clothing before reuse

Eyes: IF IN EYES, Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. If eye irritation persists: Get medical advice/attention

Ingestion: IF SWALLOWED, Call a POISON CENTER or doctor/physician if you feel unwell, Rinse mouth

Fire: In case of fire, Use CO2, dry chemical, or foam for extinction

First-aid Measures

General Advice: Show this safety data sheet to the doctor in attendance. Immediate medical attention is required.

Eye Contact: Rinse immediately with plenty of water, also under the eyelids, for at least 15 minutes. Immediate medical attention is required. Skin Contact Wash off immediately with plenty of water for at least 15 minutes. Immediate medical attention is required.

Inhalation: Move to fresh air. If breathing is difficult, give oxygen. Do not use mouth-to-mouth method if victim ingested or inhaled the substance; give artificial respiration with the aid of a pocket mask equipped with a one-way valve or other proper respiratory medical device. Immediate medical attention is required.

Ingestion: Do not induce vomiting. Call a physician or Poison Control Center immediat.

Fire-fighting Measures

Suitable Extinguishing Media: Use water spray, alcohol-resistant foam, dry chemical or carbon dioxide. Cool closed containers exposed to fire with water spray.

Unsuitable Extinguishing Media: Water may be ineffective

Flash Point: 13 °C / 55.4 °F

Autoignition Temperature: 440 °C / 824 °F

Explosion Limits:

Upper: 15.9 vol %

Lower: 6.2 vol %