Faculty of New Sciences and Technologies, University of Tehran

|  | "In the name of God" |
| --- | --- |

Instrument Safety Data Sheet

Product name: Magnesium Metal

Creation Date: 1 June 2021

Identification

Product name: Magnesium Metal

What are the hazards?

Physical hazards: Self-heating substances and mixtures - Substances and mixtures, which in contact with water, emit flammable gases.

Signal word: Danger

Hazard statement: In contact with water releases flammable gas. Self-heating; may catch fire.

Prevention: Keep away from heat/sparks/open flames/hot surfaces. No smoking. Do not allow contact with air. Wear protective gloves/protective clothing/eye protection/face protection. Keep cool. Protect from sunlight.

Response: Brush off loose particles from skin. Immerse in cool water/wrap in wet bandages.

Storage: Protect from sunlight. Maintain air gap between stacks/pallets. Store away from other materials.

Disposal: Dispose of contents/container to an appropriate treatment and disposal facility in accordance with applicable laws and regulations, and product characteristics at time of disposal.

First-aid Measures

General information: Get medical advice/attention if you feel unwell. Show this safety data sheet to the doctor in attendance.

Ingestion: Rinse mouth. Get medical attention if symptoms occur.

Inhalation: Move to fresh air. Get medical attention if symptoms persist.

Skin contact: Wash skin thoroughly with soap and water for several minutes. Get medical attention if irritation persists after washing. Wash contaminated clothing before reuse.

Eye contact: Immediately flush with plenty of water for at least 15 minutes. If easy to do, remove contact lenses. Get medical attention if irritation persists after washing.

Fire-fighting Measures

General fire hazards: In case of fire and/or explosion do not breathe fumes. Dust may form explosive mixture with air.

Suitable extinguishing media: Dry chemical.

Unsuitable extinguishing media:Water, CO2 or Foam

Specific hazards arising from the chemical: Material may react violently with water. Contact with water liberates flammable gas. Self-heating; may catch fire.