Faculty of New Sciences and Technologies, University of Tehran

|  | "In the name of God" |
| --- | --- |

Instrument Safety Data Sheet

Product name: PolymethylMethacrylate (PMMA)

Creation Date: 22 April 2021

Identification

Product name: PolymethylMethacrylate (PMMA)

Appearance and Odor: Brown liquid emulsion

Boiling Point: 100ºC / 212ºF

Glass Transition: Temp 105ºC

What are the hazards?

Eyes: Mild irritation

Skin Contact: Short exposure; no irritation. Repeated prolonged exposure, especially if confined; mild irritation, possibly a mild superficial burn.

Skin Absorption: Not likely to be absorbed in toxic amounts. Possibly weak sensitizer.

Ingestion: Low single dose toxicity.

Inhalation: No guide established. Considered to be low in hazard from inhalation.

First-aid Measures

Eyes: Flushing the eye immediately with water for 15 minutes is a good safety practice.  Physician should stain for evidence of corneal injury.

Skin: Contact may cause slight irritation. Wash off in flowing water or shower. Wash clothing before reuse. Treat as any contact dermatitis. If burn is present, treat as any thermal burn.

Ingestion: Low in toxicity. Induce vomiting if large amounts are ingested.

Inhalation: Remove to fresh air if effects occur. Consult medical personnel.

Systemic: Human effects not established. No specific antidote. Treatment based on sound  judgement of physician and the individual reactions of the patient.

Fire-fighting Measures

Extinguishing Media: Water Fog

Special Firefighting Procedures: N/A

Unusual fire and Explosion Hazards: The dried resin is flammable similar to wood. Burning dry resin emits dense, black smoke. Suspended material is not flammable.