Faculty of New Sciences and Technologies, University of Tehran

|  | "In the name of God" |
| --- | --- |

Instrument Safety Data Sheet

Product name: Sodium borohydride

Creation Date: 22 April 2021

Identification

Product name: Sodium borohydride

Recommended Use: laboratory chemical, laboratory and analytical use

What are the hazards?

Main Hazard: In contact with water releases flammable gases which may ignite spontaneously, Toxic if swallowed, Causes severe skin burns and eye damage, Harmful if inhaled, May damage fertility. May damage the unborn child (if exposed).

IF SWALLOWED: rinse mouth. Do NOT induce vomiting.

IF ON SKIN: Wash with plenty of water.

IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

First-aid Measures

General notes: Take off immediately all contaminated clothing. Self-protection of the first aider.

Inhalation: Provide fresh air. In all cases of doubt, or when symptoms persist, seek medical advice.

Skin contact: After contact with skin, wash immediately with plenty of water. Immediate medical treatment required because corrosive injuries that are not treated are hard to cure.

Eye contact: In case of contact with eyes flush immediately with plenty of flowing water for 10 to 15 minutes holding eyelids apart and consult an ophthalmologist. Protect uninjured eye.

Ingestion: Rinse mouth immediately and drink plenty of water. Call a physician immediately. If swallowed danger of perforation of the esophagus and the stomach (strong corrosive effects).

Fire-fighting Measures

Suitable extinguishing media: dry extinguishing powder, sand, cement

Unsuitable extinguishing media: water, foam, carbon dioxide (CO₂)

Special hazards arising from the substance or mixture: Combustible. The material reacts vigorously with water at ambient temperatures and demonstrates generally a tendency for the gas produced to ignite spontaneously. Vapours can form explosive mixtures with air. Material reacts vigorously with water emitting flammable gases.