Faculty of New Sciences and Technologies, University of Tehran

|  | "In the name of God" |
| --- | --- |

Instrument Safety Data Sheet

Product name: Acetic acid

Creation Date: 1 June 2021

Identification

Product name: Acetic acid

Synonyms: Ethanoic acid; Ethylic acid; Methanecarboxylic acid; Vinegar acid; Glacial acetic acid.

Appearance: clear, colorless liquid. Flash Point: 39 deg C

What are the hazards?

Danger! Causes severe eye and skin burns. Causes severe digestive and respiratory tract burns. Flammable liquid and vapor. May be harmful if absorbed through the skin. Glacial acetic acid solidifies below 62°F (17°C). Corrosive to metal.

Eye: Causes severe eye irritation. Contact with liquid or vapor causes severe burns and possible irreversible eye damage.

Skin: Causes skin burns. May be harmful if absorbed through the skin. Contact with the skin may cause blackening and hyperkeratosis of the skin of the hands.

Ingestion: May cause severe and permanent damage to the digestive tract. Causes severe pain, nausea, vomiting, diarrhea, and shock. May cause polyuria, oliguria (excretion of a diminished amount of urine in relation to the fluid intake) and anuria (complete suppression of urination). Rapidly absorbed from the gastrointestinal tract.

Inhalation: Effects may be delayed. Causes chemical burns to the respiratory tract. Exposure may lead to bronchitis, pharyngitis, and dental erosion. May be absorbed through the lungs.

Chronic: Chronic exposure to acetic acid may cause erosion of dental enamel, bronchitis, eye irritation, darkening of the skin, and chronic inflammation of the respiratory tract. Acetic acid can cause occupational asthma. One case of a delayed asthmatic response to glacial acetic acid has been reported in a person with bronchial asthma. Skin sensitization to acetic acid is rare, but has occurred.

First-aid Measures

Eyes: In case of contact, immediately flush eyes with plenty of water for a t least 15 minutes. Get medical aid immediately.

Skin: In case of contact, immediately flush skin with plenty of water for at least 15 minutes while removing contaminated clothing and shoes. Get medical aid immediately. Wash clothing before reuse.

Ingestion: If swallowed, do NOT induce vomiting. Get medical aid immediately. If victim is fully conscious, give a cupful of water. Never give anything by mouth to an unconscious person.

Inhalation: If inhaled, remove to fresh air. If not breathing, give artificial respiration. If breathing is difficult, give oxygen. Get medical aid.

Notes to Physician: Persons with pre-existing skin disorders or impaired respiratory or pulmonary function may be at increased risk to the effects of this substance. Treat symptomatically and supportively.

Fire-fighting Measures

General Information: As in any fire, wear a self-contained breathing apparatus in pressure-demand, MSHA/NIOSH (approved or equivalent), and full protective gear. During a fire, irritating and highly toxic gases may be generated by thermal decomposition or combustion. Use water spray to keep fire-exposed containers cool. Reacts with most metals to form highly flammable hydrogen gas which can form explosive mixtures with air. Flammable liquid and vapor. Vapors are heavier than air and may travel to a source of ignition and flash back. Vapors can spread along the ground and collect in low or confined areas.

Extinguishing Media: Use water spray, dry chemical, "alcohol resistant" foam, or carbon dioxide.